- Section 051000 - Structural Steel

  - PART 1  GENERAL

    - SUBMITTALS

      - Product Data:  __________.

      - Sustainable Materials Documentation: Provide documentation per Section 018113 - Sustainable Design Requirements to demonstrate compliance with Buy Clean and Buy Fair Washington Act requirements.

        - Submit a ​Sustainable Product Data Reporting Form​ for each applicable product to be used on the Project at both time of product data submittal and at project closeout.

        - Applicable Buy Clean and Buy Fair Washington Act data to be provided for the work of this Section include:

          - Required Data

            - Product quantity in Declared Units.

            - Environmental Product Declaration (EPD) which is product- and facility-specific.

          - Optional Additional Data

            - Health Product Declaration (HPD).

            - Manufacturer name and location.

            - Office of minority- and women-owned business enterprises certification.

            - Supplier code of conduct.

[Setting low embodied carbon targets is not required by the Buy Clean and Buy Fair Washington Act. Project design/construction teams can choose to add embodied carbon optimization to the project if coordinated as a project stretch goal with the State of Washington client team.

If a project team elects to build on the Act’s requirements for “Environmental Product Declarations (EPDs)” by layering on an embodied carbon optimization/reduction strategy, then the language below could be a starting place for specifying those requirements.]

        - Applicable project-specific sustainable materials goals for the work of this Section include:

          - Low Embodied Carbon Materials: Include the following additional information on the Sustainable Product Data Reporting Form:

            - Embodied carbon material baseline.

            - Actual embodied carbon of the products listed.

            - Calculations of actual project embodied carbon values for each applicable product type required for "Project Embodied Carbon Optimization" as defined in Part 2 of Section 018113 - Sustainable Design Requirements.

      - Shop Drawings:  Indicate ​__________​.

  - PART 2  PRODUCTS

    - PERFORMANCE REQUIREMENTS

[The Buy Clean and Buy Fair Washington Act does require EPDs for all “covered product types”. The basic EPD ask should always be included in specifications covering these materials.]

      - Materials Transparency - Environmental Product Declaration (EPD): For each product, provide copies of all available current product disclosures. At a minimum, products must provide at least one of the following:

        - Product and facility-specific EPD: Third-party verified, current, and specifically for the product and the final facility where it was manufactured or fabricated.

        - Product and facility-specific EPD with supply chain-specific data: Third-party verified, current, and specifically for the product, its final manufacturing facility, and its supply chain.

[The Buy Clean and Buy Fair Washington Act does not require embodied carbon reductions. Setting baselines and optimization/reduction targets are optional additions that can be added to projects.]

      - Embodied Carbon - Structural Steel

        - Provide a Type III, Product and Facility-Specific Environmental Product Declaration (EPD) that documents the following:

          - Actual Embodied Carbon: Product's GWP value as listed on EPD in kg CO2eq per Declared Unit.

          - System Boundary: Product Stage A1-A3.

          - Declared Unit: Metric ton.

          - Product Category Rule (PCR): PCR used to create EPD, as listed on EPD.

[The Buy Clean and Buy Fair Washington Act does not require embodied carbon reductions. Setting baselines and optimization/reduction targets are optional add that can be added to projects.]

        - Embodied Carbon Optimization Targets: EPDs for these product types are required to have a Global Warming Potential (Embodied Carbon, Actual) of [80%] [insert project target] Embodied Carbon Baseline values or less.

[The following baselines can be found in the 2023 Carbon Leadership Forum, North American Material Baselines, Baseline Report v2, dated August 2023.  This reference can be found online at: https://carbonleadershipforum.org/clf-material-baselines-2023/Forum.]

        - Embodied Carbon Material Baselines: As expressed in kg CO2eq per Declared Unit.

          - Hot-rolled structural steel:

            - Unfabricated:      1000

            - Fabricated:           1220

          - Hollow structural steel

            - Unfabricated:      1710

            - Fabricated:           1990

          - Metal deck:                   2320

          - Structural steel plate

            - Unfabricated:      1480

            - Fabricated:           1730

[Provisions of the Buy Clean and Buy Fair Washington Act criteria that are optional (listed as “when available” below) should be researched and edited to align with project-specific goals, product selections, and market availability.]

      - Materials Transparency - Material ingredients: When available, provide copies of all unexpired Health Product Declaration (HPD) documents.

      - Materials Transparency - MWBE certification: When available, provide an unexpired Office of minority- and women-owned business enterprises (MWBE) certification document.

      - Materials Transparency - Supplier Code of Conduct: When available, provide an unexpired Supplier Code of Conduct document.

        - Any of the following types are acceptable:

          - A building product company's Supplier Code of Conduct which addresses each of the ILO Fundamental Principles and Rights at Work.

          - A Design for Freedom Supplier Questionnaire specifically completed for the product and its supply chain.

          - Any standard listed as acceptable for LEED v4.1 Pilot Credit IPpc144 - Social Equity within the Supply Chain.

- END OF SECTION